10/100Mbps 4+2 Port 250m PoE Switch

with built-in Power Supply (ND1P402F)

Feature

16-Pot 10/100Mbps +2G TP/SFP Combo

PoE Switch with LCD Display

# √ 4*10/100Mbps PoE ports, 2*10/100Mbps uplink port;

√ Support IEEE802.3af/at;

√ Support restrain network storm, VLAN function;

√ Support Max Transmission distance up to 250m with PoE Power;

√ All port supported by MDI/MDIX auto flip and self-negotiation;

√ Built-in power supply;

√ Maximum power of single PoE port: 30W;

√ Maximum PoE power: 75W;

√ Adopts store-and-forward architecture;

√ Fanless, natural cooling, small,compact and quiet design, suitable for desktop or wall;

√ Support lightning protection 6000V;

Technical Data sheet

| Power Adapter Voltage |  | AC 110V-240V |
| --- | --- | --- |
| Consumption |  | 75W(52V 1.44A) |
| Network Port | PoE Ethernet Port | 1-4 port:10/100Mbps |
|  | Ethernet Port | Uplink port:10/100Mbps |
| Transmission Distance |  | 100Mbps:0-100m 10Mbps:0-250m |
|  |  | Uplink Port:0-100m |
| Transmission Medium |  | Cat5/5e/6 standard network cable |
| Network Standard |  | IEEE802.3/802.3u,IEEE802.3x,IEEE802.3ab |
| Switching Capacity |  | 1.2Gbps |
| Packet Buffer Memory |  | 1Mbits |
| Throughput |  | 0.9Mbps |
| MAC Table |  | 1K |
| Packet Forwarding rate |  | 10Mbps:14880pps/port 100Mbps:1488000pps/port |
| PoE Standard |  | IEEE802.3af/at |
| PoE Power Supply |  | 12 45 +,36 78 -,8 core power supply |
| PoE Power |  | af=15.4w,at=30w for each port |
| Port Function |  | Power priority mechanism,fast and forward, MAC automatic learning and aging IEEE802.3X Full-duplex and mode and backpressure for Half-duplex mode |
| LED Indicator |  | LINK/ACT.100Mbps;PoE Status Indicator; Power indicator;Extender |
| Working temperature |  | 0℃~50℃ |
| Relative Humidity |  | 20-95% |
| Storage temperature |  | -20℃~70℃ |
| Dimension(L*W*H) |  | 202mm*140mm*45mm |
| Color |  | Black |
| Weighr |  | 0.95kg |

Diagram

Dimensions

Shenzhen Newbridge Communication Equipment CO.,LTD

Tel：+86 755 85292270		Email:info@newbridge.com.cn		Website:www.newbridge.com.cn